Engineering Recommendation M31

Issue 2.6 July 2026

Strategy for Supplier population of ESME Network Operators’ Trust Anchor Cells

© 2025 Energy Networks Association

All rights reserved. No part of this publication may be reproduced, stored in a retrieval system or transmitted in any form or by any means, electronic, mechanical, photocopying, recording or otherwise, without the prior written consent of Energy Networks Association. Specific enquiries concerning this document should be addressed to:

Electricity Systems Directorate
Energy Networks Association
4 More London Riverside
London
SE1 2AU

This document has been prepared for use by members of the Energy Networks Association to take account of the conditions which apply to them. Advice should be taken from an appropriately qualified engineer on the suitability of this document for any other purpose.

First published, July 2017

Amendments since publication

| Issue | Date | Amendment |
| --- | --- | --- |
| 1 | July 2017 | First Publication |
| 1.1 | March 2018 | Updated details for SPEN MANW area |
| 1.2 | July 2018 | Updated UKPN MPID and Cert details |
| 1.2a | July 2018 | Updated first page to show correct issue date as “July 2018” and added note on page 6 that all updated credentials are to be implemented with immediate effect. |
| 1.3 | February 2019 | Updated document to show Energetics MPID and Certificate details |
| 1.5 | May 2020 | Addition of credentials for Energy Assets Networks in table on page 7. Addition of credentials for UK Power Distribution in table on page 7. Correction to Harlaxton certificate Subject Name in table on page 6. New paragraph at the end of page 5. |
| 1.5 | July 2020 | UK Power Distribution (iDNO) provided updated credentials as their original details were found to be incorrect. |
| 1.6 | July 2021 | Credentials entered for Fulcrum Electricity Assets, Leep Electricity Networks Ltd, Energy Assets Pipelines Ltd, Eclipse Power Ltd and Murphy Power Distribution Ltd. |
| 1.7 | September 2021 | Updated details for Eclipse Power Ltd |
| 1.8 | September 2022 | Updated Details for MUA Electricity Limited (previously Murphy Power Distribution Limited) |
| 2.0 | November 2025 | Updated document in readiness for upcoming updates |
| 2.1 | January 2026 | Updated Details for GTC - Published DNO Certificates ENC001/IPN001 |
| 2.2 | February 2026 | Updated details for Indigo Power Limited added. |
| 2.3 | April 2026 | Updated details for National Grid (WPD) SWAE 21 region. |
| 2.4 | June 2026 | Updated details for National Grid (WPD) SWEB 22 region. |
| 2.5 | July 2026 | Removed details of Harlaxton until new certificate details received. |
| 2.6 | July 2026 | Updated details for National Grid (WPD) MIDE 14 region. Updated details for SP Electricity North West NORW 16 region. |

Contents

1	Purpose of this document	4

2	Scope of this document	4

3	Introduction and background	4

4	Guidance for suppliers	4

# Purpose of this document

At the time of installation and commissioning of a new electricity smart meter a supplier has an obligation under the Smart Energy Code (SEC) to retrieve the appropriate DNO or iDNO Organisation Certificates from the SMKI Repository and use it to populate the Network Operator’s Trust Anchor Cell within the meter.  This document provides the necessary guidance for suppliers, setting out the correct electricity network operator credentials to be used during installation and commissioning of new meters that are to be enrolled into DCC systems.

# Scope of this document

The scope of this document is limited to the provision of guidance to suppliers on the appropriate use of the given DNO and iDNO credentials in accordance with Inventory, Enrolment and Withdrawal Procedures.

# Introduction and background

During 2016 DNOs received a number of requests from Suppliers and their representatives querying how Suppliers will be able to determine which DNO Organisation Certificates (assuming that for an individual DNO network there is more than one) should be retrieved from the SMKI Repository and placed in the Network Operators’ Trust Anchor Cells in an ESME at the time of installation and commissioning.

These queries are driven by the Suppliers’ obligation in the Smart Energy Code (Clause 4.19 of the SEC Appendix AC "Inventory, Enrolment and Decommissioning Procedures') that reads as follows:

4.19 As soon as reasonably practicable (and in any event within 7 days) following the Commissioning of a Smart Meter, a Standalone Auxiliary Proportional Controller or a Gas Proxy Function, the Responsible Supplier shall, in relation to each such Device, ensure that:

- the Device Security Credentials which pertain to the Network Parties are: (a) (b) in the case of those which pertain to the Electricity Network Party, those of the Electricity Distributor;

- and in the case of those which pertain to the Gas Network Party, those of the Gas Transporter (or, if they are not available in the SMKI Repository, the Device Security Credentials which pertain to the Gas Network Party shall be populated with information from the Organisation Certificates Issued to the DCC and referred to in Sections L3.23(f) (the DCC (access-Control-Broker) – digital Signature Certificate) and L3.23(g) (the DCC (access-Control-Broker) – key Agreement Certificate));

This document seeks to clarify the position from a DNO perspective to inform suppliers and enable them to fulfil their SEC obligations clause 4.19.

# Guidance for suppliers

For the purposes of the Supplier obligation DNOs have agreed that Suppliers should select any valid Organisation Certificates for that DNO to place in the Trust Anchor Cells during or shortly post installation and commissioning. DNOs accept that the onus is then on the DNO to change their Organisation Certificates on that ESME, should they wish to do so. DNOs accept that it will be for them to manage any additional complexity that arises from adopting this approach.

The DCC User Interface Specification (DUIS) in SEC Appendix AC "Inventory, Enrolment and Decommissioning Procedures' specifies that User Role Signing Key pairs must not be used for communication with Devices. The relevant extract from the DUIS is replicated below:

3.3.1 DUIS XML Service Request Signing

The User shall Digitally Sign every XML format Service Request and Signed Pre-Command sent to the DCC using a User Role Signing Private Key. This must be a separate dedicated Key that shall not be used for communication with Devices (i.e. different to that used to sign the GBCS Payload held within Signed Pre-Commands). A separate User Role Signing Private Key must be used per User Id in use for each User.”

Each User shall notify the DCC of the Organisation Certificate corresponding to each User Role Signing Private Key that they wish to use for Digitally Signing communications to the DCC in accordance with the paragraph above.

In the event that a Supplier inadvertently obtains the certificate relating to the User Role Signing Private Key and places this on a device in the Network Operator Anchor Slot then this may be construed as a breach of the SEC. Suppliers should therefore use the DNO credentials provided in the table below in accordance with Inventory, Enrolment and Withdrawal Procedures.

Where DNO credentials are not provided in the table below, Suppliers should leave the Network Operator Trust Anchor field as 'ACB'."

ENA has also agreed that any independent network operator company can provide their credentials for inclusion in this document. Any iDNO details that have been provided have been added to the table below.

Suppliers should use the following DNO/iDNO credentials in accordance with Inventory, Enrolment and Withdrawal Procedures:

| DNO Name | MPID | Distribution Area | Party Signifier | Certificate Subject Name | Certificate Serial (Digital Signature) | Certificate Serial (Key Agreement) | Expiry Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SP Electricity North West | NORW | 16 | ENW001 | 70-B3-D5-1F-30-1C-80-01 | 4C334CD88561A9191ED93229893F63AC | 232D9239C018B99614DE444EE68C3304 |  |
| WPD | EMEB | 11 | EME001 | 70-B3-D5-1F-30-7D-AA-AA | 162ea3fbc9792e0c21acc7560efeb6ec | 292918f9af5ae08a44b0802c4b135b01 |  |
| WPD | SWAE | 21 | SWA001 | 70-B3-D5-1F-30-7E-AA-AA | 65eccb4eb95eeb9183e02b357e9775bb | 528dc0317eee54a85f1f91a110a9881e |  |
| WPD | SWEB | 22 | SWE001 | 70-B3-D5-1F-30-7F-AA-AA | 76C760152139A2448B2AA783D2E96037 | 0EC0DF2003C61BACD9106188A4FA0897 |  |
| WPD | MIDE | 14 | MID001 | 70-B3-D5-1F-30-80-AA-AA | 62003058144B9189AA8D4942DC3B6B78 | 7464A92790A13E209518349C30AECB29 |  |
| UKPN (EPN) | EELC | 10 | PHB009 | 70-B3-D5-1F-30-17-00-00 | 2de3d09aca79f55daf1abbab5c8dc4c7 | 3730424e52409cd52abb22e3df276ce2 |  |
| UKPN (SPN) | SEEB | 19 | PFG007 | 70-B3-D5-1F-30-65-00-00 | 40038c78f5b21bdc627c0480aec2953c | 4df989c931ec99cb01515e15dd7424f7 |  |
| UKPN (LPN) | LOND | 12 | PBE008 | 70-B3-D5-1F-30-41-00-00 | 75553f2558294f0fbd1b175875a1c470 | 5fcbf0b276ec5d57efbea4d299380a11 |  |
| UKPN Olympic Park iDNO | EDFI | Not Used | N/A | No longer used | No longer used | No longer used |  |
| NPG | NEEB | 15 | NPG001 | 70-B3-D5-1F-30-48-00-00 | 5eb83995d8569b5883e8f897adbcf527 | 419c3b2a8ee830b58ca801fd0dbecce2 |  |
| NPG | YELG | 23 | NPG002 | 70-B3-D5-1F-30-49-00-00 | 5b6889789f1db1705dbac28d1d591e94 | 3009cc21ac86dd2c66dac96853decd73 |  |
| SPEN | SPOW | 18 | SPD001 | 70-B3-D5-1F-30-6A-30-60 | 4371D41B2FB7789F7C6325BCC9AB15D2 | 2E201886CF7028950AD35F9A54647066 |  |
| SPEN | MANW | 13 | SPD002 | 70-B3-D5-1F-30-6B-30-60 | 62C97583733C9F240F590E11923C1A1D | 7f15bcc4ef1d0aeb41254d84545b084f |  |
| SSEN | SOUT | 20 | SED001 | 70-B3-D5-1F-30-68-80-01 | 1120a4a31f208d04e59ed0d676dcdf99 | 34e192500d4c35bcc6cb56ddeb63baf5 |  |
| SSEN | HYDE | 17 | SHD001 | 70-B3-D5-1F-30-5E-80-01 | 6115bd7cfcc0339abef649c50ed48cbb | 61df87da2dcd1fffebacd99b9452d670 |  |
| GTC | ETCL | 27 | ENC001 | 70-B3-D5-1F-30-70-00-01 | 47c0025e7a449ddac19504a9dd151f98 | 7bb73f4bffa678cb355c7533a8341232 |  |
| GTC | IPNL | 24 | IPN001 | 70-B3-D5-1F-30-39-00-01 | 2f3b150ab4bea5c2d763cca317d4140e | 6831f80fed063b63a7898185ba77706c |  |
| Energetics | GUCL | 26 | ENE001 | 70-B3-D5-1F-30-89-00-20 | 7ADA985832603B14B2DF93F582E823A5 | 202272E75DE2986D0F6D88A26E6F47B8 |  |
| ESP Electricity | LENG | 25 | ESP005 | 70-B3-D5-1F-30-1E-00-AB | 3fd398496fb0d4ed3ef3bcc8c1b502d6 | 4d7ca796b3e67b31030e4d377ad503d6 |  |
| DNO Name | MPID | Distribution Area | Party Signifier | Certificate Subject Name | Certificate Serial (Digital Signature) | Certificate Serial (Key Agreement) | Expiry Date |
| Energy Assets Networks | UDNL | 32 | PEP002 | 70-B3-D5-1F-30-F4-00-01 | 294345F1A2317CEAB5DC8B5D0AD1284C | 115F67F088E5A830DDF65C19305A0AC5 |  |
| UK Power Distribution | UKPID | 31 | PPL009 | 70 B3 D5 1f 30 BB 00 10 | 6a930b40bad7492739632b3516b54234 | 5ff278b11a236bae20c573cd07b728db |  |
| Energy Assets Pipelines Ltd | EAP | 77 | PDN008 | 70-B3-D5-1F-30-D3-00-01 | 17A6179EA2D574DBB9F72DCA999D989A | 7A46BB2D1335B90D51C7D7B2E05ABE88 |  |
| Fulcrum Electricity Assets Ltd | FEAL | 35 | PLC013 | 70-B3-D5-1F-31-2B-00-00 | 59F249EEEBD31AFB10588C3D1CA2BA7F | 697CFE7AF7CAEB27F17811D67003ED8A |  |
| LEEP Electricity Networks Limited | PENL | 30 | PCM000 | 70-B3-D5-1F-30-C2-30-00 | 475BAA670C2483A8F9E5C8CCBB9F97CB | 3F054F83CD7471A5BE34129995B53F95 |  |
| Eclipse Power Ltd | GGEN | 33 | PBC017 | 70-B3-D5-1F-31-21-00-25 | 523B13D99074AD97E106B84337919EC3 | 5CAA4A283A9ADA81FFB39C1F1976E1CB |  |
| MUA Electricity Limited (previously Murphy Power Distribution Limited) | MPDL | 34 | PFD010 | 70-B3-D5-1F-31-35-00-30 | 5CF29DD42FF6CC92979096CB3F7E9692 | 1416351D85EC2C1CBD1368CFF2320C53 |  |
| Indigo Power Limited | INDI | 38 | PJH011 | 70-B3-D5-1F-31-79-00-25 | 5F19E519059AC06B0BC96AB26FC0F681 | 1dd361e9884e5745da96b6d021786004 |  |

NOTE: All updates shown in the table above are to be implemented with immediate effect.

—————————